Thiamine Analysis Protocol – PSEC Fish Nutrition Lab

By Bill Chen  2025.12 v1

Adopted from Dr. Jacques Richard’s lab at Suny Brockport and Cody Pinger’s lab at NOAA

1. Reagents Needed

- 2% trichloroacetic acid (TCA)

Dissolve 20 g TCA solids in 1 L filtered de-ionized (DI) water [2% w/v]

[SUNY: 20 ml of TCA in 980 ml of RO water and store in refrigerator]

- 25 mM potassium phosphate buffer (pH 8.4)

|  | To make 2 L buffer | To make 1 L buffer |
| --- | --- | --- |
| KH2PO4 Potassium monobasic phosphate | 0.1928 g | 0.964 g |
| K2HPO4 Potassium dibasic phosphate | 8.464 g | 4.232 g |

To make 2 L buffer, dissolve 0.1928 g KH2PO4 and 8.464 g K2HPO4 in 1.9 L of filtered DI water

Adjust the pH with 1 M HCl or 1 M NaOH to pH 8.4;

Bring up the total volume to 2 L in a volumetric flask;

Filter the phosphate buffer with 0.2 μm Nylon Net Filters in a magnetic funnel

- 0.1% K3FeCN6, potassium ferric cyanide

0.01 g of K3FeCN6 in 10 ml of filtered DI water

Prepare solution daily and protect from light by wrapping it with aluminum foils

- 1N NaOH, sodium hydroxide

20 g of NaOH in 500 ml of filtered DI water, and store in refrigerator [SUNY]

(*NUT Lab has been using NaOH at room temperature with no issue)

- Ethyl acetate:hexane (3:2, volume per volume)

Mix 300 mL Ethyl acetate with 200 mL hexane

- N,N-dimethylformamide, HPLC grade

- Methanol, ultrapure, HPLC Grade

-de-ionized water filtered through 0.2 μm Nylon Net Filters

Thiamine Standards Stock Solution Preparation

TH - Thiamine hydrochloride (MW = 337.26 g/mol) (Sigma, P/N T4562), stored at 4 oC

TMP - Thiamine monophosphate chloride dihydrate (MW = 416.8 g/mol) (Sigma, P/N T8637), stored at

4 oC

TPP - Thiamine pyrophosphate chloride or cocarboxylase (MW = 460.8 g/mol) (Sigma, P/N C8754), stored -20 oC

- To make 1.0 mM (1.0 mmol/L) solution of the standards, dissolve the following amounts of thiamine compounds in corresponding amount of water

| Compound | Amount of thiamine compounds (g) |  |  |
| --- | --- | --- | --- |
|  | In 1000 mL Water | In 500 mL Water | In 250 mL Water |
| TH | 0.33726 | 0.16863 | 0.08432 |
| TMP | 0.4168 | 0.2084 | 0.1042 |
| TPP | 0.4608 | 0.2304 | 0.1152 |

- Cover the solution to protect it from light

- Aliquot 1 mL of the standard solutions into 1.5 mL centrifuge tubes (labelled with compound name and date), can keep 30 – 50 tubes per compound;

- Freeze and keep all the standards tubes in -80 freezer

- To avoid degradation due to light and temperature, finish the aliquot step as soon as possible;

- Do not prepare all three 1.0 mM solutions all at once; finish freezing all tubes for 1 compound first, then move on to prepare the solutions and repeat the steps

2. Protocol

Thiamine is light sensitive. Therefore, its extraction and dosage should be performed under low light intensity environment. Also keep the samples on ice whenever possible.

Part I Extraction of Thiamine

- Keep the 15 mL centrifuge tubes (for tissue homogenization), and labelled 1.5 mL centrifuge tubes (for extract) in the -20 oC; keep these tubes cold before being used

- Fill the dark-covered boxes with ice

- Start boiling water in 1 L beakers on a hot plate – it will take time

- Calculate and pour out the 2% trichloroacetic acid (TCA) needed in a beaker; kept on ice

- Weigh approximately 1 g  [0.8 – 1.2 g] of fish egg (around 2 g for other tissues) directly in 15 mL centrifuge tube – record weight and count fish egg number – keep each sample tube on ice in a dark-covered box

- Add 6 beads (cold, kept at -20oC) into each sample tube, and keep the sample tubes on ice inside a covered box

- In a dark room, using a multi-dispense pipette, add 6 ml of cold 2% TCA (6 ml for eggs, 4 ml for other tissues)

- Homogenize the samples with beads mill homogenizer, max 12 samples per round [total duration: 50 s, speed: 4.0 m/s, time: 20 s per cycle, 2 cycles total, dwell: 10 s] – fully tighten the caps for each tube, use the black plastic bag to cover the homogenizer, and also check to see if any samples are spilled to the plastic cover of the machine

- Put the 15 mL centrifuge tubes with homogenized samples into the boiling water bath, Incubate tubes in a water bath at 90-100oC for 10 min – use aluminum foils to cover the water bath and the samples

- Cool tubes on ice in a black-covered box for 15 min

- Centrifuge tubes for 15 min (centrifuge in room 127;  3360 g, set temperature at 4oC or lower)

- Use disposable glass Pasteur pipets, transfer the extract from each sample tube into a set of 2 x 1.5 mL centrifuge tubes – only take the clear layer of exact, do not take the solids at the bottom or the oil on the top; take majority of the exact when possible;

- Freeze tubes with extract in biofreezer at -80oC

***Recover the beads from the sample waste

-use a big spatula to break up the sample waste at the bottom of the tubes

-pour out everything into a strainer – both the sample waste and beads will be in the strainer

-use water to rinse to get rid of most sample waste, and then recover the beads

-use hot water and soap to clean the beads

-dry the beads and keep them in a 50 mL centrifuge tube in -20oC freezer

***be careful not to lose beads into the drain

Part II Standard Preparation and Cleaning of Extract (done on ice)

*DO NOT let standards or samples freeze and thaw

2.1 Thiamine standard Preparation

- Cover and thaw the 3 thiamine standards (each at 1 mmol/L) on ice or at room temperature

- BEFORE the standards are completely thawed, vortex them to finish the thawing process

- In a 1.5 mL Eppendorf tube,

To make a 300 μmol/L of each vitamer stock A solution:

add 300 μL of each vitamer + 100 μL of H2O to make up 1000 μL (1 mL); vortex

To make a 200 μmol/L of each vitamer stock B solution:

add 200 μL of each vitamer + 400 μL of H2O to make up 1000 μL (1 mL); vortex

- Do a serial 1:1 dilutions of stock B (200 μmol/L) to prepare the rest of standards at 100, 50, 25, 12.5, 6.25 μmol/L

- Label 15-mL centrifuge tubes with 30, 20, 10, 5, 2.5, 1.25, 0.625, and 0 nmols; add 6 mL of 2% TCA into each tube; kept the tubes on ice

- Add 100 μL of each stock standard into the corresponding tubes containing 2% TCA – add 100 μL water into the tube for 0 nmols

- Pipet 600 μL of thiamine standards in a 1.5 mL Eppendorf tube

- Add 600 μL of ethyl-acetate:hexane solution

- Vortex Eppendorf tube for 3 – 5 seconds

- Remove upper layer after it settles down using pipetter

- Repeat steps 4 to 6, and wash the standards with ethyl-acetate:hexane for a total of 4 times

- Transfer 425 microliter of the washed standard solution to a micro-centrifugal filter tube [750 µL, PTFE, 0.20 µm]; kept all tubes on ice

- On ice, add 45 microliter of NaOH (1N) and 30 microliter of 0.1% potassium ferricyanide simultaneously into each filter tube

- Centrifuge the micro-centrifuge filter tubs at 8000 g for 5 mins

- Make sure all exact are filtered through, discard the filter and transfer all extract in HPLC vials

- Load extract in HPLC vials and run them via HPLC

2.2 Cleaning of thiamine extract

- Remove the homogenate tubes from biofreezer and let them thaw on ice in a dark-covered box or on table (covered by aluminum foils]

*It usually take at least 30 – 40 mins to thaw at room temperature

- Vortex each tube to make sure they are completely thawed

- Centrifuge each sample tube for 30 s to separate any solids in the extract

- Pipet 600 μL of thiamine extract in a 1.5 mL Eppendorf tube

- Add 600 μL of ethyl-acetate:hexane solution

- Vortex Eppendorf tube for 3 – 5 seconds

- Remove upper layer after it settles down using disposable glass Pasteur pipets

- Repeat steps 4 to 6, and wash the homogenate with ethyl-acetate:hexane for a total of 4 times (this step removes TCA and increase pH to ~5)

- Transfer 425 microliter of the washed aqueous solution to a micro-centrifugal filter tube [750 µL, PTFE, 0.20 µm]; kept all tubes on ice

- On ice, add 45 microliter of NaOH (1N) and 30 microliter of 0.1% potassium ferricyanide simultaneously into each filter tube

- Centrifuge the micro-centrifuge filter tubs at 8000 g for 5 mins

- Make sure all exact are filtered through, discard the filter and transfer all extract in HPLC vials

- Load extract in HPLC vials and run them via HPLC

HPLC  Methods

Part 1: Solvent Purge [run this before running analysis]

Flow rate: 0.5 mL/min; run for 45 min

For flushing out the methanol inside the HPLC capillary tubes

| Time (min) | A (%) 25 mM phosphate buffer | B (%) DMF - Dimethylformamide |
| --- | --- | --- |
| 0 | 100 | 0 |
| 15 | 0 | 100 |
| 30 | 100 | 0 |
| 45 | 100 | 0 |

Part 2: Thiamine analysis

Flow rate: 1.0 mL/min; run for 20 mins

Based on the paper “thiamine analysis in fish tissues” published by Brown et al., 1998

| Time (min) | A (%) 25 mM phosphate buffer | B (%) DMF - Dimethylformamide |
| --- | --- | --- |
| 0 | 100 | 0 |
| 4 | 100 | 0 |
| 4.1 | 97 | 3 |
| 8 | 97 | 3 |
| 8.1 | 80 | 20 |
| 13 | 80 | 20 |
| 13.1 | 100 | 0 |
| 20 | 100 | 0 |

Part 3: Shutdown method [run this before shutting down]

Flow rate: 0.5 mL/min; run for 120 min

For flushing out anything (ex. mobile phase, thiamine) left inside the capillary tube; need to do this especially if you know the HPLC is not going to be used for a while after the previous run;

Important for maintaining the column

| Time (min) | A (%) Filtered DI Water | B (%) HPLC grade methanol |
| --- | --- | --- |
| 0 | 100 | 0 |
| 14.5 | 100 | 0 |
| 15 | 95 | 5 |
| 29.50 | 95 | 5 |
| 30 | 85 | 15 |
| 44.50 | 85 | 15 |
| 45 | 70 | 30 |
| 59.50 | 70 | 30 |
| 60 | 50 | 50 |
| 79.50 | 50 | 50 |
| 80 | 30 | 70 |
| 99.50 | 30 | 70 |
| 100 | 0 | 100 |
| 120 | 0 | 100 |

List of Supplies

TH - Thiamine hydrochloride (MW = 337.26 g/mol) (Sigma, T4562) - $94 for 25 g

TMP - Thiamine monophosphate chloride dihydrate (MW = 416.8 g/mol) (Sigma, T8637) - $86 for 5 g

TPP - Thiamine pyrophosphate chloride (MW = 460.8 g/mol) (Sigma, C8754) - $93 for 5 g

K2HPO4 - di-Potassium hydrogen phosphate ACS (VWR, BDH9266-500G) - $105 for 500 g

KH2PO4 - Potassium dihydrogen phosphate ≥99.5%, HiPerSolv CHROMANORM® for HPLC (VWR, BDH153184U) - $75 for 500 g

N,N-Dimethylformamide ≥99.9%, HiPerSolv CHROMANORM® for HPLC (VWR, BDH83634.400) - $272 for 4 L

Methanol, ultrapure, HPLC Grade, 99.8+% (Scientific, AA22909K7) - $190 for 4 L

750 μL Nonsterile Micro-Centrifugal Filters, PTFE, 0.2 μm, (Chromspec, C58F25179  or ThermoFisher, F2517-9)  - $375 for 100 ps

Polypropylene Vials (Agilent, 5191-8121) - $18 for 100

Polypropylene Vials Caps (Agilent, 5191-8151) - $12 for 100

15 mL Basix™ Polypropylene Conical Centrifuge Tubes (Fisher Scientific, 14955237) - $110 for 500

Fisherbrand™ Disposable Borosilicate Glass Pasteur Pipets (Fisher Scientific, 13-678-20D) - $121 for 1440 ps

MilliporeSigma™ Nylon Net Filters, 0.2 μm, (Fisher Scientific, GNWP04700) - $240 for 100 ps

500 mL Magnetic Filter Funnels, 47 mm, Cytiva (Formerly Pall Lab) (VWR, CA28150-496) - $ 431 each

HPLC

HPLC Column - PRP-1, 5 µm, 4.1 x 150 mm (Hamilton, PN: 79444) - $750 for 1

Guard Column - Analytical Guard Cartridges, PRP-1, 2.1 x 20 mm, stainless steel  (Hamilton, PN: 79445) - $ 305 for 5

Guard Column Cartridge Holder, Stainless Steel (Hamilton, PN: 32908) - $166 for 1

Homogenizer

Bead Ruptor EliteTM Bead Mill Homogenizer, Omni International, (Revvity, 19-042E) - $16,000

Bead Ruptor Elite 15 mL Tube Carriage & Finger Plate (Revvity, 19-375) - $3020

Stainless Steel Beads Bulk, 5 mm, 325 g (Revvity, 27-7005-3) - $406